4

5

Final evaluation (switching model)

7 controllers × 7 scenarios (49 simulations per case): overshoot, settling time, margins

Statistical analysis

best / mean / std / worst of J, success rate, convergence, Wilcoxon rank-sum test

Outputs: selected PI gains (Kp, Ki), robustness tables and figures, design guidelines

Re-ranking on the switching-level Simulink model

best 3 candidates per optimizer, 3 optimization scenarios (27 simulations per case); the best is retained

averaged model

switching model

cost / analysis

setup / outputs

30 independent runs per optimizer (seeds 1-30) · Case 1:

Kp ≥ 0, Case 2: Kp ≥ 0.001 · population 12, 20 iterations

1

2

3

Optimization on the averaged small-signal model

J

Averaged model Gvd(s):

step response and loop-gain margins

Cost formulation

metrics per scenario: ITAEn, overshoot, Ts,n, duty eßort E, with penalties on phase margin (< 45◦) and gain margin (< 6 dB)

robust scalar cost J = (1 − λ) Jk + λ maxk Jk	| vector cost F for NSGA-II

Baseline designs

Manual (PID Tuner app) Calculated (crossover at fz/10)

PID Tuner (pidtune, nominal plant)

Converter and operating scenarios

500 W boost converter, 12 V—→24 V—, fs = 10 kHz 7 scenarios: Vin ± 20%, load 50/20 %, L, C −20%

optimization set Sopt: nominal, Vin − 20%, load 20 %

NSGA-II

GA

GWO

PSO