# Manuscript Sections: Results, Discussion, Conclusions and Back Matter

Drafting notes (delete before submission). Section numbers assume 1 Introduction, 2 Converter modelling, and 3 Proposed tuning methodology. Items in [square brackets] need your input. All numerical values come from your simulation logs; the reference list at the end must be verified before use. Figure numbers follow the order of your Word file (Figs. 1–5 = Case A, Figs. 6–9 = Case B).

## 4. Results

### 4.1 Evaluation protocol

The framework was evaluated on a 500 W boost converter (Vin = 12 V, Vo = 24 V, fs = 10 kHz, L = 1.44 mH, C = 4.34 mF, R = 1.152 Ω) modelled in MATLAB/Simulink with switching semiconductor devices and a PWM generator (Figure 1) A single voltage-mode PI controller, Cs= Kp+Kis, acts on the output-voltage error. All tests are start-up responses to a 24 V reference from zero initial conditions over a 0.4 s horizon.

Figure 1- Simulink model of the boost converter with voltage-mode PI control, PWM generator and logging blocks.

Seven operating scenarios were defined by perturbing the nominal point (Table 1): input voltage ±20 %, load reduced to 50 % and 20 % of rated power, and inductance and capacitance reduced by 20 %. Table 1 also lists the averaged-model characteristics of each scenario, namely the DC gain G₀ = V_in/D′², the right-half-plane (RHP) zero frequency f_z = R D′²/(2πL), the resonant frequency f₀ = D′/(2π√(LC)) and the quality factor Q = D′R√(C/L), where D′ = 1 − D. Three scenarios (nominal, V_in − 20 % and 20 % load) formed the optimisation set; the remaining four were held out and used only for validation.

Table 1 Operating scenarios and averaged-model characteristics D = 1 -VinVo . † Marks scenarios used inside the optimisation cost.

| Scenario | Perturbation | D | G₀ (V) | f_z (Hz) | f₀ (Hz) | Q |
| --- | --- | --- | --- | --- | --- | --- |
| Nominal † | none | 0.50 | 48.0 | 31.8 | 31.8 | 1.00 |
| V_in − 20 % † | V_in = 9.6 V | 0.60 | 60.0 | 20.4 | 25.5 | 0.80 |
| V_in + 20 % | V_in = 14.4 V | 0.40 | 40.0 | 45.8 | 38.2 | 1.20 |
| Load 50 % | R = 2.304 Ω | 0.50 | 48.0 | 63.7 | 31.8 | 2.00 |
| Load 20 % † | R = 5.760 Ω | 0.50 | 48.0 | 159.2 | 31.8 | 5.00 |
| L − 20 % | L = 1.152 mH | 0.50 | 48.0 | 39.8 | 35.6 | 1.12 |
| C − 20 % | C = 3.472 mF | 0.50 | 48.0 | 31.8 | 35.6 | 0.89 |

Performance was quantified by (i) the normalised integral of time-weighted absolute error, ITAE_n = ∫ t|e| dt / (V_ref T_stop²/2), which equals 1 for a response that never leaves zero; (ii) the peak overshoot; (iii) the 2 % settling time T_s; and (iv) the duty-cycle effort, defined as the total variation of the duty ratio sampled once per switching period so that PWM ripple does not contribute. Gain margin (GM) and phase margin (PM) were computed from the averaged small-signal model. The optimisation cost combined these terms into a weighted sum evaluated over the optimisation scenarios, blending the weighted mean (weight 0.5) and the worst case (weight 0.5), and added penalties when PM < 45° or GM < 6 dB.

Four reference designs were compared with the metaheuristic designs: (i) Manual, gains chosen by trial and error (K_p = 0.0132, K_i = 0.6867); (ii) Calculated, an analytical design with the crossover at one tenth of the nominal RHP-zero frequency and the PI zero at five times the crossover (K_p = 0.0040, K_i = 0.4045); (iii) PID Tuner, automatic tuning of the Simulink PID Tuner on the nominal averaged plant (K_p = 0, K_i = 0.8837); and (iv) the designs obtained by PSO, grey wolf optimisation (GWO) and a genetic algorithm (GA). Each optimiser was run 30 times with independent random seeds, using a population of 12 and 20 iterations (at most 240 cost evaluations per run). The 30 runs were executed on the averaged model for tractability; the three best solutions of each optimiser were then re-ranked on the switching Simulink model, and all time-domain values reported below were obtained from the switching model. Two search spaces were studied: Case A, K_p ∈ [0, 0.03], which lets the optimiser choose freely, and Case B, K_p ∈ [0.001, 0.03], which enforces a genuine PI structure. K_i ∈ [0.1, 2] in both cases. Multi-objective tuning used NSGA-II with a population of 40 and 30 generations.

### 4.2 Case A: robust tuning without a lower bound on Kp

Worst-case performance. Table 2 summarizes the worst case over all seven scenarios. The two designs tuned for the nominal point or by hand were not robust. The Manual design reached 18.7 % overshoot at 20 % load and did not settle within the 0.4 s horizon in that scenario. The PID Tuner design, although the fastest at the nominal point, produced 13.2 % overshoot at V_in − 20 % and 15.2 % overshoot at 20 % load, and also failed to settle at 20 % load. In the averaged model both designs had at least one scenario with an unstable closed loop. The Calculated design was stable in all scenarios, with a worst-case overshoot of 0.63 %, a worst-case settling time of 288.8 ms and a minimum phase margin of 33.7°.

The PSO, GWO and GA designs converged to practically the same controller: K_p = 0 and K_i between 0.4086 and 0.4090, an integral-only law. Relative to the Calculated design, the PSO design reduced the worst-case ITAE_n from 0.0598 to 0.0553 (−7.6 %) and the worst-case settling time from 288.8 ms to 254.3 ms (−12.0 %), and it raised the minimum phase margin from 33.7° to 67.5° and the minimum gain margin from 4.98 dB to 5.85 dB. The price was a small increase in worst-case overshoot, from 0.63 % to 0.76 %. Compared with the Manual and PID Tuner designs, the worst-case overshoot fell by factors of about 25 and 20, respectively. The PSO design satisfied the 45° phase-margin target with a wide margin, while its gain margin (5.85 dB) remained marginally below the 6 dB soft target; this is a deliberate consequence of the trade-off with settling speed discussed in Section 5.

Table 2 Worst-case performance over seven scenarios, Case A (K_p ≥ 0). NS = not settled within 0.4 s; "unstable" = at least one scenario unstable in the averaged model.

| Controller | K_p | K_i | ITAE_n,max | Overshoot_max (%) | T_s,max (ms) | PM_min (°) | GM_min (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Manual | 0.0132 | 0.6867 | 0.0887 | 18.68 | NS (>400) | unstable | unstable |
| Calculated | 0.0040 | 0.4045 | 0.0598 | 0.633 | 288.8 | 33.7 | 4.98 |
| PID Tuner | 0 | 0.8837 | 0.0766 | 15.21 | NS (>400) | unstable | unstable |
| PSO | 0 | 0.4086 | 0.0553 | 0.758 | 254.3 | 67.5 | 5.85 |
| GWO | 0 | 0.4090 | 0.0551 | 0.760 | 254.0 | 67.5 | 5.84 |
| GA | 0 | 0.4087 | 0.0552 | 0.758 | 254.2 | 67.5 | 5.85 |
| NSGA-II (knee) | 0.0036 | 0.3047 | 0.1038 | 0.350 | 373.6 | 85.5 | 7.11 |

Scenario-wise behaviour. Table 3gives overshoot and settling time for each scenario. The PSO design settled 9.3–15.7 % faster than the Calculated design in every scenario (for example 239.2 ms against 269.9 ms at the nominal point and 248.5 ms against 288.8 ms at 20 % load), while its overshoot remained below 0.8 % everywhere. The Calculated design had slightly lower overshoot than PSO in all seven scenarios, with differences of at most 0.15 percentage points. The spread of the PSO settling times across scenarios was narrow, from 203.9 ms to 254.3 ms. In the four held-out scenarios (V_in + 20 %, 50 % load, L − 20 %, C − 20 %) the PSO design gave overshoots between 0.21 % and 0.66 % and settling times between 242.6 ms and 254.3 ms. The GWO and GA designs were indistinguishable from the PSO design at the resolution of the tables (overshoot differences below 0.02 percentage points and settling-time differences below 0.4 ms), and are therefore not tabulated separately.

Table 3 Overshoot (%) / settling time (ms) per scenario, Case A. NS = not settled within 0.4 s.

| Scenario | Manual | Calculated | PID Tuner | PSO | NSGA-II (knee) |
| --- | --- | --- | --- | --- | --- |
| Nominal | 0.53 / 186.4 | 0.44 / 269.9 | 0.63 / 95.7 | 0.52 / 239.2 | 0.00 / 361.2 |
| V_in − 20 % | 1.16 / 169.3 | 0.63 / 241.8 | 13.24 / 391.3 | 0.76 / 203.9 | 0.35 / 328.7 |
| V_in + 20 % | 0.63 / 189.5 | 0.20 / 280.5 | 0.44 / 109.1 | 0.35 / 254.3 | 0.00 / 373.6 |
| Load 50 % | 0.98 / 221.5 | 0.11 / 271.7 | 0.38 / 113.2 | 0.21 / 244.2 | 0.00 / 358.5 |
| Load 20 % | 18.68 / NS | 0.08 / 288.8 | 15.21 / NS | 0.10 / 248.5 | 0.00 / 363.9 |
| L − 20 % | 0.53 / 189.9 | 0.42 / 274.0 | 0.54 / 101.4 | 0.51 / 244.1 | 0.00 / 365.3 |
| C − 20 % | 0.67 / 190.9 | 0.57 / 274.4 | 0.81 / 96.7 | 0.66 / 242.6 | 0.00 / 367.4 |

Start-up response. Fig. 4 shows the nominal start-up response of all controllers. All designs share an initial transient in which the output rises to roughly 17–20 V within about 10 ms and then dips to 10–12 V near 17 ms, which is consistent with the excitation of the converter's open-loop LC resonance during start-up; the controller gains therefore mainly shape the slower, integral-dominated approach to 24 V. The PID Tuner design reached the reference in about 100 ms, the Manual design showed the largest initial peak (about 20 V) and a visible secondary oscillation, and the PSO, GWO and GA responses overlap and approach 24 V monotonically without visible overshoot at this scale. The NSGA-II knee design was the slowest and the most heavily damped.

### 4.3 Reliability of the optimisers (Case A)

All three optimisers attained the same best cost, J = 0.6261, so none was able to find a better solution than the others; they differed in how reliably they reached it (Table 4, Figs. 2 and 3). PSO reached the best cost, to within 0.1 %, in all 30 runs (success rate 100 %), with a mean cost of 0.6261 and a standard deviation below 10⁻⁴. GWO reached it in 10 of 30 runs (33.3 %), with a mean of 0.6386 and a standard deviation of 0.0120. GA reached it in 7 of 30 runs (23.3 %), with a mean of 0.6602, a standard deviation of 0.0426 and a worst-case cost of 0.8103. The Wilcoxon rank-sum test (two-sided, Bonferroni-corrected significance level 0.05/3 = 0.0167) showed that the final-cost distribution of PSO differed significantly from those of GWO and GA (p < 0.001 in both comparisons), whereas GWO and GA did not differ significantly from each other (p ≈ 0.25).

The convergence curves (Fig. 2, mean best cost over 30 runs) show that PSO started at a low cost (about 0.9) because its initial swarm contained the baseline gains, whereas GWO started from random positions at a mean cost near 8, presumably because several initial candidates were unstable and therefore heavily penalised. Both algorithms settled within 0.1 % of their final cost after a similar number of iterations (8.6 for PSO and 7.9 for GWO on average). The GA convergence history was not recorded.

### 4.4 Multi-objective tuning (Case A)

NSGA-II was used to explore the trade-off between speed (ITAE_n + T_s + margin penalty), worst-case overshoot and duty-cycle effort. The resulting Pareto front (Fig. 5) contained about a dozen non-dominated designs, all with worst-case overshoot below 0.4 % and duty effort between roughly 0.38 and 0.47, and with the speed index spanning approximately 0.6 to 1.2, so that no single design minimised all three objectives. The knee point, selected as the solution closest to the normalised utopia point, had K_p = 0.0036 and K_i = 0.3047. It reduced the worst-case overshoot to 0.35 % and raised the margins to PM_min = 85.5° and GM_min = 7.11 dB, which satisfies both stability-margin targets, but its worst-case settling time rose to 373.6 ms, which is 47 % longer than that of the PSO design. NSGA-II was run once, so the front is illustrative and no statistical statement is made about it.

Table 4. Statistical comparison of the optimisers over 30 independent runs on the final cost J. Success = runs within 0.1 % of the best cost found. p-values: two-sided Wilcoxon rank-sum test. [Replace the Case A p-values with the output of MATLAB's ranksum before submission.]

| Case | Optimiser | Best | Mean | Std | Worst | Success | Mean iterations to converge |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A (K_p ≥ 0) | PSO | 0.6261 | 0.6261 | < 10⁻⁴ | 0.6262 | 30/30 (100 %) | 8.6 |
|  | GWO | 0.6261 | 0.6386 | 0.0120 | 0.6536 | 10/30 (33.3 %) | 7.9 |
|  | GA | 0.6261 | 0.6602 | 0.0426 | 0.8103 | 7/30 (23.3 %) | n/a |
| B (K_p ≥ 0.001) | PSO | 0.6615 | 0.6615 | < 10⁻⁴ | 0.6616 | 30/30 (100 %) | 7.5 |
|  | GWO | 0.6615 | 0.6617 | 0.0003 | 0.6626 | 26/30 (86.7 %) | 8.2 |
|  | GA | 0.6617 | 0.7458 | 0.0821 | 0.9326 | 1/30 (3.3 %) | n/a |

Wilcoxon p-values. Case A: PSO vs GWO < 0.001; PSO vs GA < 0.001; GWO vs GA ≈ 0.25. Case B: PSO vs GWO = 8.3 × 10⁻⁶; PSO vs GA = 3.0 × 10⁻¹¹; GWO vs GA = 6.7 × 10⁻¹¹.

### 4.5 Case B: a genuine PI controller (K_p ≥ 0.001)

Because the unconstrained optimum in Case A degenerates to an integral controller, the search was repeated with a lower bound K_p ≥ 0.001 so that the result remains a PI controller. All three optimisers placed K_p at or very near the bound (0.0010–0.0011) with K_i between 0.4136 and 0.4164 (Table 5). Relative to the Calculated design, the PSO design reduced the worst-case ITAE_n by 9.1 % (0.0598 to 0.0544) and the worst-case settling time by 11.3 % (288.8 ms to 256.1 ms), lowered the worst-case overshoot slightly below that of the unconstrained design (0.652 % against 0.758 %), and gave a minimum phase margin of 70.7° and a minimum gain margin of 5.68 dB. Imposing the bound increased the optimised cost by 5.7 % (0.6261 to 0.6615), which quantifies the price of insisting on a non-zero proportional term. The per-scenario behaviour (Table 6) mirrored Case A: the PSO design stayed below 0.7 % overshoot with settling times between 208.4 ms and 256.1 ms, while the Manual and PID Tuner designs failed at 20 % load, and the PID Tuner design also at V_in − 20 %.

The optimiser statistics were consistent with Case A and more pronounced for GA (Table 4, Figs. 6 and 7). PSO reached the best cost in 30 of 30 runs, GWO in 26 of 30 (86.7 %) and GA in only 1 of 30 (3.3 %), with a GA mean cost of 0.7458 and a worst cost of 0.9326. All pairwise differences were significant (p < 0.0167). GA solutions showed K_p values up to 0.004, which the cost penalises through reduced gain margin and slower settling, indicating that its search did not consistently reach the corner of the feasible region where the optimum lies. The NSGA-II knee (K_p = 0.0041, K_i = 0.2885; Fig. 9) reduced the worst-case overshoot to 0.058 % and kept GM_min at 7.11 dB, but the worst-case settling time reached 399.5 ms, essentially the whole simulation horizon, and PM_min fell to 45.1°; this solution should therefore be viewed as a conservative design rather than a practical one.

Table 5. Worst-case performance over seven scenarios, Case B (K_p ≥ 0.001). Reference designs are unchanged from Table 2.

| Controller | K_p | K_i | ITAE_n,max | Overshoot_max (%) | T_s,max (ms) | PM_min (°) | GM_min (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Manual | 0.0132 | 0.6867 | 0.0887 | 18.68 | NS (>400) | unstable | unstable |
| Calculated | 0.0040 | 0.4045 | 0.0598 | 0.633 | 288.8 | 33.7 | 4.98 |
| PID Tuner | 0 | 0.8837 | 0.0766 | 15.21 | NS (>400) | unstable | unstable |
| PSO | 0.0010 | 0.4147 | 0.0544 | 0.652 | 256.1 | 70.7 | 5.68 |
| GWO | 0.0010 | 0.4136 | 0.0547 | 0.652 | 256.9 | 70.7 | 5.70 |
| GA | 0.0011 | 0.4164 | 0.0540 | 0.653 | 255.4 | 70.8 | 5.63 |
| NSGA-II (knee) | 0.0041 | 0.2885 | 0.1164 | 0.058 | 399.5 | 45.1 | 7.11 |

Table 6. Overshoot (%) / settling time (ms) per scenario, Case B. NS = not settled within 0.4 s.

| Scenario | Manual | Calculated | PID Tuner | PSO | NSGA-II (knee) |
| --- | --- | --- | --- | --- | --- |
| Nominal | 0.53 / 186.4 | 0.44 / 269.9 | 0.63 / 95.7 | 0.52 / 242.3 | 0.00 / 387.4 |
| V_in − 20 % | 1.16 / 169.3 | 0.63 / 241.8 | 13.24 / 391.3 | 0.64 / 208.4 | 0.06 / 355.0 |
| V_in + 20 % | 0.63 / 189.5 | 0.20 / 280.5 | 0.44 / 109.1 | 0.34 / 256.1 | 0.00 / 399.5 |
| Load 50 % | 0.98 / 221.5 | 0.11 / 271.7 | 0.38 / 113.2 | 0.21 / 246.8 | 0.00 / 384.1 |
| Load 20 % | 18.68 / NS | 0.08 / 288.8 | 15.21 / NS | 0.10 / 247.5 | 0.00 / 388.9 |
| L − 20 % | 0.53 / 189.9 | 0.42 / 274.0 | 0.54 / 101.4 | 0.51 / 247.0 | 0.00 / 391.4 |
| C − 20 % | 0.67 / 190.9 | 0.57 / 274.4 | 0.81 / 96.7 | 0.65 / 245.9 | 0.00 / 394.3 |

## 5. Discussion

### 5.1 Why the optimum is integral-dominant

The results are explained by the structure of the boost-converter power stage rather than by the choice of optimizer. In continuous conduction the control-to-output transfer function of the averaged model is

Gvds= VinD'2·1 -sLRD'21 +sLR D'2+s2LCD'2,

which is flat at the DC gain V_in/D′² (48 at the nominal point) up to a resonance at f₀ and contains an RHP zero at f_z. For the converter studied here the two frequencies coincide at the nominal operating point (f_z = f₀ = 31.8 Hz, Q = 1.0), and the resonance becomes sharply underdamped at light load (Q = 5.0 at 20 % load, Table 1). Because the plant gain does not roll off before the resonance, a proportional term raises the loop gain at and above the resonance, where the phase is already close to −180° (integrator lag plus resonant lag plus RHP-zero lag), and therefore consumes gain margin without providing useful bandwidth. Integral action, in contrast, sets a low crossover below the lowest RHP-zero frequency (20.4 Hz at V_in − 20 %). The optimised gain K_i ≈ 0.409 corresponds to a crossover of approximately K_i·G₀ ≈ 19.6 rad/s (3.1 Hz), about one tenth of the nominal RHP-zero frequency; this agrees with the classical rule of placing the crossover well below f_z and explains why the optimiser drove K_p to its lower bound.

The data support this interpretation. At almost equal K_i (0.405–0.415), increasing K_p from 0 (Case A PSO) to 0.001 (Case B PSO) to 0.004 (Calculated) lowered the minimum gain margin monotonically from 5.85 dB to 5.68 dB to 4.98 dB and lengthened the worst-case settling time from 254.3 ms to 256.1 ms to 288.8 ms. The robustness failures of the Manual and PID Tuner designs are explained by the same mechanism. A first-order estimate of the loop gain at the resonance for an integral controller is |L(jω₀)| ≈ (K_i/ω₀)·G₀·Q, which at 20 % load gives about 0.49 for the PSO design (K_i = 0.409) but about 1.06 for the PID Tuner design (K_i = 0.884). A value above one at a frequency where the phase is near −180° implies a lightly damped or unstable loop, consistent with the 15 % overshoot and the absence of settling observed for the PID Tuner design in the switching simulation. This estimate neglects the RHP-zero phase lag and is intended only as an order-of-magnitude explanation.

### 5.2 Robustness versus speed

The comparison exposes a clear trade-off. Tuning for the nominal point alone, as the PID Tuner does, gives the fastest nominal response (95.7 ms against 239.2 ms for PSO, i.e. about 2.5 times faster), but the resulting design is not robust: it fails at two of the seven scenarios. The robust designs sacrifice nominal speed to guarantee acceptable behaviour everywhere, with a narrow settling-time band (204–254 ms) and overshoot below 0.8 %. The multi-objective results show that the trade-off can be shifted further toward robustness, as with the NSGA-II knee design, but only at the cost of settling times of 330–400 ms. The appropriate design depends on the application: where the load varies widely and start-up overshoot must be avoided, the robust PSO or Calculated designs are preferable; where operation is confined to the nominal point, a faster design could be justified provided the operating range is enforced.

It should also be noted that the analytically derived design is itself a strong baseline. It was stable in all scenarios and had lower overshoot than the PSO design in all seven. The contribution of the optimisation is therefore best described as a systematic refinement of an already sound analytical design (about 12 % faster settling, a doubling of the minimum phase margin and a small gain in gain margin), together with a demonstration of how fragile designs tuned at a single operating point are, rather than the discovery of a radically different controller.

### 5.3 Behaviour of the optimisers

The three algorithms found the same best solution, which is expected for a two-parameter problem in which the optimum lies on the boundary of a smooth, low-dimensional landscape. What differed was reliability. PSO converged to the optimum in every run in both cases, GWO did so in one third of the runs in Case A and in most runs in Case B, and GA did so rarely. A plausible explanation is that the cost surface is non-smooth, with a steep penalty cliff for unstable gains and a narrow, flat valley adjacent to the K_p bound; a population-based method with velocity memory, such as PSO, handles such a landscape well, while the default crossover and mutation operators of GA tend to produce candidates scattered across the bound. Both PSO and GWO converged in a similar number of iterations (7.5–8.6), so the advantage of PSO lies in the consistency of the final result rather than in speed.

These observations must be interpreted with caution. First, the PSO and GA populations were seeded with the three baseline designs, one of which (Calculated) lies within 1 % of the optimal K_i, whereas GWO started from random positions; this gives the seeded algorithms an advantage that was not isolated here. The fact that GA was seeded and still performed worst suggests that seeding does not explain the whole difference, but an unseeded comparison would be needed to confirm this. Second, the population size and number of iterations were kept small (at most 240 evaluations) to make 30 repetitions affordable, and the GA operators were left at their default values; a tuned GA could perform better. Third, the problem has only two decision variables, so the ranking should not be generalised to higher-dimensional controller structures.

### 5.4 Practical design implications

Three practical guidelines follow from the results. First, for a single-loop voltage-mode PI controller on a boost converter operating in continuous conduction, the controller should be integral-dominant, with the crossover frequency of the order of one tenth of the lowest RHP-zero frequency over the expected operating range; a larger proportional term degrades the gain margin without improving the response. Second, tuning should consider the worst-case operating point, which for this converter is light load (highest Q) combined with low input voltage (lowest f_z), rather than the nominal point. Third, the analytical design procedure provides an excellent starting point and a useful seed for metaheuristic refinement, which can then be used to adjust the compromise between speed, overshoot and stability margins through the cost-function weights. If a strictly positive proportional gain is required by the controller implementation, the penalty of enforcing K_p ≥ 0.001 was small (a 5.7 % increase in cost) and the resulting design retained nearly all of the robustness benefits.

### 5.5 Limitations and threats to validity

The following limitations should be considered when interpreting the results.

- The optimisation runs used the averaged small-signal model, and the switching-model Simulink simulation was used only to re-rank the best candidates and to produce the reported results. The agreement between the averaged and switching models in ranking the three baseline controllers at the nominal point supports this approach, but the absolute values differ (for example, the averaged model overestimates overshoot), and optimisation directly on the switching model might yield slightly different gains.

- All tests were reference-step (start-up) responses. Load-step and line-step transients, which are the usual regulation tests, were not included and should be added in future work.

- The two most demanding scenarios (V_in − 20 % and 20 % load) were part of the optimisation set, and the four held-out scenarios are comparatively benign for all controllers except the Manual and PID Tuner designs; the evidence of generalisation is therefore limited, and simultaneous perturbations (for example low input voltage together with light load) were not examined.

- Gain and phase margins were computed from the averaged model assuming continuous conduction, unity PWM gain and ideal components, and the model may be inaccurate at light load if the converter enters discontinuous conduction.

- The cost-function weights and the margin thresholds were chosen heuristically and affect the compromise between speed and robustness; the reported Pareto front illustrates this sensitivity but was obtained from a single NSGA-II run.

- The study is simulation-only; no hardware or hardware-in-the-loop validation was performed, and parasitic effects (inductor and capacitor resistance, semiconductor losses, sensing delay) were limited to those in the Simulink device models.

- The settling-time values are capped by the 0.4 s simulation horizon; designs reported as not settled may settle later, and the settling time of the NSGA-II knee in Case B (399.5 ms) is at the edge of the horizon.

## 6. Conclusions

This paper presented a robustness-aware tuning framework for the PI voltage controller of a boost converter, in which the gains are obtained by particle swarm optimisation over a set of operating scenarios using a multi-term cost (ITAE, overshoot, settling time, duty-cycle effort and stability-margin penalties), and compared it with manual tuning, an analytical design, the Simulink PID Tuner, grey wolf optimisation, a genetic algorithm and NSGA-II. The main conclusions are as follows.

- Controllers tuned manually or by automatic tuning at the nominal operating point were not robust. Over seven scenarios, the manual and PID Tuner designs produced 18.7 % and 15.2 % worst-case overshoot and failed to settle at 20 % load, while the PSO-tuned design kept the worst-case overshoot at or below 0.76 % and the worst-case settling time at 254 ms.

- Relative to a well-designed analytical controller, PSO tuning reduced the worst-case settling time by about 11–12 % and the worst-case ITAE by 8–9 %, roughly doubled the minimum phase margin (33.7° to 67.5–70.7°) and modestly improved the minimum gain margin, at the cost of a small increase in overshoot (below 0.15 percentage points).

- The unconstrained optimum is an integral controller (K_p = 0). This follows from the coincidence of the RHP zero and the LC resonance, which makes proportional action detrimental to the gain margin. Enforcing a true PI structure (K_p ≥ 0.001) increased the cost by only 5.7 % and preserved the robustness benefits.

- In 30 independent runs, PSO reached the best cost in every run for both search spaces, whereas GWO did so in 33 % and 87 % of runs and GA in 23 % and 3 % of runs; the differences between PSO and the other optimisers were statistically significant. All three algorithms attained the same best solution, so the difference lies in reliability and not in the attainable solution quality.

- The multi-objective analysis showed a trade-off between speed and robustness: designs with the highest margins and lowest overshoot required settling times of 330–400 ms.

Future work will include load-step and line-step tests, validation of the switching model against a hardware prototype or a hardware-in-the-loop platform, optimisation directly on the switching model, a cascaded voltage–current control structure that is not limited by the RHP zero, an unseeded and budget-matched optimiser comparison with tuned algorithm parameters, and the evaluation of additional metaheuristics.

## Figure captions

- Fig. 2. Convergence of PSO and GWO (mean best cost over 30 runs), Case A (K_p ≥ 0).

- Fig. 3. Distribution of the final cost over 30 runs for PSO, GWO and GA, Case A.

- Fig. 4. Nominal start-up response of the output voltage for all controllers, Case A.

- Fig. 5. NSGA-II Pareto front (speed index, worst-case overshoot, duty-cycle effort), Case A.

- Fig. 6. Convergence of PSO and GWO (mean best cost over 30 runs), Case B (K_p ≥ 0.001).

- Fig. 7. Distribution of the final cost over 30 runs for PSO, GWO and GA, Case B.

- Fig. 8. Nominal start-up response of the output voltage for all controllers, Case B.

- Fig. 9. NSGA-II Pareto front, Case B.

## Back matter (edit and complete)

Author contributions (CRediT). [Conceptualisation, Methodology, Software, Validation, Formal analysis, Investigation, Writing – original draft, Writing – review and editing: names/initials.]

Funding. [State funding sources or "This research received no external funding."]

Declaration of competing interest. [The authors declare no competing interests.]

Data and code availability. The Simulink model, MATLAB scripts and raw results (results_A_Kp0.mat, results_B_Kp001.mat) are available at [repository link/DOI, for example Zenodo or GitHub].

Acknowledgements. [Optional.]

## References (verify every entry before use; add the papers from your literature review)

[1] J. Kennedy and R. Eberhart, "Particle swarm optimization," in Proc. IEEE Int. Conf. Neural Networks, 1995, pp. 1942–1948.

[2] Z.-L. Gaing, "A particle swarm optimization approach for optimum design of PID controller in AVR system," IEEE Trans. Energy Convers., vol. 19, no. 2, pp. 384–391, Jun. 2004.

[3] R. W. Erickson and D. Maksimović, Fundamentals of Power Electronics, 3rd ed. Cham, Switzerland: Springer, 2020.

[4] R. D. Middlebrook and S. Ćuk, "A general unified approach to modelling switching-converter power stages," in Proc. IEEE Power Electronics Specialists Conf. (PESC), 1976, pp. 18–34.

[5] S. Mirjalili, S. M. Mirjalili and A. Lewis, "Grey wolf optimizer," Adv. Eng. Softw., vol. 69, pp. 46–61, 2014.

[6] K. Deb, A. Pratap, S. Agarwal and T. Meyarivan, "A fast and elitist multiobjective genetic algorithm: NSGA-II," IEEE Trans. Evol. Comput., vol. 6, no. 2, pp. 182–197, Apr. 2002.

[7] F. Wilcoxon, "Individual comparisons by ranking methods," Biometrics Bull., vol. 1, no. 6, pp. 80–83, 1945.

[8] [Deželak et al., "PI Controllers Performance of a Grid-Connected Solar PV System with PSO and Ziegler–Nichols Tuning," Energies, vol. 14, no. 9, art. 2516, 2021. Add volume/pages after checking, and cite it in your Introduction as related work.]